1.Make Folder for the Project e.g. Pioneer_Control

2.Copy all files from this folder to Pioneer_Control

Path for files-

C:\Program Files\CoppeliaRobotics\CoppeliaSimEdu\programming\legacyRemoteApi\remoteApiBindings\python\python

3.Copy this file into Pioneer_Control (It has files for MacOs and Ubuntu) select according to OS.

Path for file-

Windows

C:\Program Files\CoppeliaRobotics\CoppeliaSimEdu\programming\legacyRemoteApi\remoteApiBindings\lib\lib\Windows

4.Open CoppeliaSim. Save First Scene eg. First_test in the folder Pioneer_Control.

5. Add child lua script to the any object in the scene. Eg. To the Floor.

6.Open that script and paste following for the connection.

simRemoteApi.start(19999)

7. Write Code in python (eg. First_test_code) and save it in the same folder (eg. Pioneer_Control).

Following code provides connection between python and CoppeliaSim

sim.simxStart('127.0.0.1', 19999, True, True, 5000, 5)

Complete code-

import sim

import sys

print("Program Started")

sim.simxFinish(-1)

clientID = sim.simxStart('127.0.0.1', 19999, True, True, 5000, 5)

if(clientID != -1):

print('Connected Successfully.')

else:

sys.exit('Failed To connect.')

8.Run the Simulation in the CoppeliaSim

9.Run the Python script

Example to Run basic code and control Pioneer 3DX in the CoppeliaSim using python.

10.Example script to run the Pioneer robot.

I have added scene “Pioneer_control” and the script “Control_robot.py” for your reference.

Before running this script Disable the original Pioneer script in the CoppeliaSim

Code-

import sim

import time

import sys

print("Program Started")

sim.simxFinish(-1) #CLose the previous connection

clientID = sim.simxStart('127.0.0.1', 19999, True, True, 5000, 5) # Establish the connection

if(clientID != -1):

print('Connected Successfully.')

else:

sys.exit('Failed To connect.')

time.sleep(1)

#Get the object handle for the motors

error_code, left_motor_handle = sim.simxGetObjectHandle(clientID, '/PioneerP3DX/leftMotor', sim.simx_opmode_oneshot_wait)

error_code, right_motor_handle = sim.simxGetObjectHandle(clientID, '/PioneerP3DX/rightMotor', sim.simx_opmode_oneshot_wait)

#Give the commands to the robot about velocity

error_code = sim.simxSetJointTargetVelocity(clientID, left_motor_handle, 0.4, sim.simx_opmode_oneshot_wait)

error_code = sim.simxSetJointTargetVelocity(clientID, right_motor_handle, 1, sim.simx_opmode_oneshot_wait)

Script explanation

Import Sim

Get object handle eg. right motor

Set velocity

Documentation Link

https://manual.coppeliarobotics.com/en/remoteApiFunctionsPython.htm